FTP001_V1

Date: 03/02/2020

| TECHNICAL DATA SHEET OF ACRYLIC DOORS WITH AGLOMERATED INTERIOR |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BUILDING | DOOR TYPE | Interior swing flat door. |  |  |  |  |  |  |  |  |  |  |
|  | PERIMETER FRAME | Spruce pine section 30x25mm (in 35mm blade) |  |  |  |  |  |  |  |  |  |  |
|  | INSIDE | Multilayer wood particle board, bonded together by aminoplast resins with the simultaneous action of pressure and heat, and subsequently subjected to surface sanding. |  |  |  |  |  |  |  |  |  |  |
|  | INTERIOR PROPERTIES | Humidity |  |  |  | EN 322 |  |  |  | 5% - 13% |  |  |
|  |  | Formaldehyde content |  |  |  | EN 120 |  |  |  | Class E1 (≤8mg/100g) |  |  |
|  |  | Thick swelling |  |  |  | EN 317 |  |  |  | ≤10% |  |  |
|  |  | Absorption |  |  |  | EN 317 |  |  |  | ≤50% |  |  |
|  | PARAMENTS | Medium density fibreboard (650-700 kg / m3) 5 / 7mm thick with a decorative polypropylene-based decorative sheet finish. |  |  |  |  |  |  |  |  |  |  |
|  | EXTERIOR FINISH PROPERTIES | Chemical resistance |  |  | DIN 68861/1 |  |  |  |  |  |  | Class 1C |
|  |  | Abrasion resistance |  |  | Taber abrasor CS 10 |  |  |  |  |  |  | Min. 400 turns |
|  |  | Dry heat |  |  | DIN 68861/7 |  |  |  |  |  |  | Class 7C (=100°C) |
|  |  | Humid heat |  |  | DIN 68861/8 |  |  |  |  |  |  | Class 8C (=100°C) |
|  |  | Light resistance |  |  | ISO 4892-2 |  |  |  |  |  |  | ≥5 |
|  | EDGE | 0.8mm thick PVC (variable thickness according to finishes) |  |  |  |  |  |  |  |  |  |  |
| MOLDINGS | Swinging material (when appropriate) | Water-resistant or standard MDF according to request. Density 650-700kg / m3 |  |  |  |  |  |  |  |  |  |  |
|  | BATIENTE FINISH (when appropriate) | Foil finish similar to leaf finishing. |  |  |  |  |  |  |  |  |  |  |
|  | PLUG MATERIAL (when appropriate) | Water-resistant or standard MDF according to request. Density 650-700kg / m3 |  |  |  |  |  |  |  |  |  |  |
|  | COVER FINISH (when appropriate) | Foil finish similar to leaf finishing. |  |  |  |  |  |  |  |  |  |  |
|  | CORE MOLD PROPERTIES (waterproof finish) | Humidity |  |  |  | EN 322 |  |  | 4%-11% |  |  |  |
|  |  | Density toleracy |  |  |  | EN 323 |  |  | ±7% |  |  |  |
|  |  | Formaldehyde content |  |  |  | EN 120 |  |  | E1 ≤ 8mg/100g |  |  |  |
|  |  |  |  |  |  |  |  |  | E2 ≤ 30mg/100g |  |  |  |
| SHEET | CORE MOLD PROPERTIES S/UNE EN 1529 | Squad | ±1,5mm |  |  |  |  |  |  |  | (class 1) |  |
|  |  | Height and width | ± 2mm |  |  |  |  |  |  |  | (class 1) |  |
|  |  | Thick | ±1,5mm |  |  |  |  |  |  |  | (class 1) |  |
|  | PLANITUDE S/UNE EN 1530 | Warp | ≤4mm |  |  |  |  |  |  |  | (class 3) |  |
|  |  | Curvature beams. | ≤4mm |  |  |  |  |  |  |  | (class 3) |  |
|  |  | Curvature heads | ≤2mm |  |  |  |  |  |  |  | (class 3) |  |
|  |  | Local flatness | ≤0,3mm |  |  |  |  |  |  |  | (class 3) |  |
|  | WATER DIVING S/UNE 56 803 | Swelling |  |  |  |  | 1,2% |  |  |  |  |  |
|  |  | Observations |  |  |  |  | No decoupling |  |  |  |  |  |
|  | SCREW STARTING S/UNE 56 803 | Middle value (≥500N) |  | Results |  |  |  | Paraments: 1144N |  |  |  |  |
|  |  | Minimum value (≥500N) |  |  |  |  |  | Edge: 1376N |  |  |  |  |

*See detail of the door on the next page

FTP001_V1

Date: 03/02/2020

TECHNICAL DATA SHEET OF ACRYLIC DOORS WITH AGLOMERATED INTERIOR

Signed:

Jesús Bejo Santiago

Quality manager